Checklist of 36 Actual Measurements

Measurements taken on level ground in an unladen state in accordance with GB 1589-2016 and GB/T 3730.3; registration inspection tolerance: ±1% or ±50 mm, whichever is stricter.

I. Overall Vehicle Dimensions (8 items)

1. Overall vehicle length (including bumpers, excluding side mirrors)

2. Overall vehicle width (excluding side mirrors and protruding side marker lights)

3. Overall vehicle height (unladen, excluding openable roof hatch)

4. Front overhang: Distance from the center of the front axle to the foremost point of the vehicle

5. Rear overhang: Distance from the center of the rearmost axle to the rearmost point of the vehicle

6. First wheelbase: Distance between the centers of the 1st and 2nd axles

7. Second wheelbase: Distance between the centers of the 2nd and 3rd axles (for multi-axle vehicles)

8. Third wheelbase: Distance between the centers of the 3rd and 4th axles (for four-axle vehicles)

I. Wheel assembly off-ground (6 items)

- 1. Front wheel track (distance between tire centers)

- 2. Rear wheel track (drive axle)

- 3. Minimum ground clearance (lowest point of the chassis)

- 4. Height of front wheel center from the ground

- 5. Height of rear wheel center from the ground

- 6. Height of the frame's top surface from the ground (average of front, middle, and rear points)

I. Cab-related items (5 items)

- 1. Cab length (from outer surface of front windshield to outer surface of rear cab panel)

- 2. Cab width (maximum overall width)

- 3. Cab height (from top surface of frame to cab roof)

- 4. Distance from rear cab panel to center of first rear axle

- 5. Distance from rear cab panel to rear end of frame

I. Cargo Box / Superstructure Exterior (7 items)

- 1. Overall external length of the cargo box

- 2. Overall external width of the cargo box

- 3. Overall external height of the cargo box (to the top of the sideboards)

- 4. Distance from the outer face of the front sideboard to the center of the front axle

- 5. Distance from the outer face of the rear sideboard to the rear end of the chassis frame

- 6. Left-side overhang of the cargo box (relative to the chassis frame)

- 7. Right-side overhang of the cargo box (relative to the chassis frame)

I. Cargo compartment interior (4 items; mandatory for official announcement; average value based on measurements at front, middle, and rear points) — Ministry of Industry and Information...

- 1. Cargo bed internal length

- 2. Cargo bed internal width

- 3. Cargo bed internal height (side panel height)

- 4. Cargo bed floor height from the ground

I. Protective and Safety Devices (6 items)

- 1. Ground clearance of the lower edge of the front bumper

- 2. Ground clearance of the lower edge of the rear underrun protection

- 3. Distance from the left and right ends of the rear protection to the outer edge of the rear tires

- 4. Ground clearance of the left side protection (at front, middle, and rear points)

- 5. Ground clearance of the right side protection (at front, middle, and rear points)

- 6. Clearance between the front and rear ends of the side protection and the outer edge of the tires